Příloha č. 1 Smlouvy: Technická specifikace předmětu plnění

Předmětem dodávky je zařízení: Ultrazvukové svařovací zařízení, které umožňuje spojovat plastové materiály pomocí ultrazvukové technologie. Výhodou je nižší tepelné namáhání.

Splňuje následující (minimální) technické parametry a umožňuje následující funkce:

- frekvence ultrazvukového generátoru 35 kHz;

- výkon ultrazvukového generátoru minimálně 1000 W;

- svařovací přítlak v rozmezí 30 – 600 N;

- zdvih (Stroke) 100 mm;

- dotykový barevný displej minimálně 14″;

- servopneumatický pohon válce (pneumatický pohon válce s možností nastavení startovní a koncové pozice válce);

- svařovací módy – možnost řízení svařovacího profilu dle Energie, Výkonu, Času, Absolutní vzdálenosti, Hloubky či kombinace jednotlivých módů během jednoho svařovacího profilu;

- možnost nastavení profilu minimálně se třemi různými přítlaky během procesu svařování a minimálně jedním přítlakem před a po ukončení svařování;

- systém odměřování dráhy;

- grafické zobrazení svařovacího profilu i výsledku svařovacího procesu na barevném displeji;

- možnost uložení nastavených profilů svařování (minimálně 200 profilů);

- možnost záznamu průběhu svařovacího procesu (minimálně 200 profilů);

- možnost porovnání průběhů svařování (minimálně 100 průběhů);

- sledování kvality procesu svařování s možností grafické analýzy (pomocí metrik CPK a regulačních diagramů);

- české komunikační rozhraní;

- dvouruční mechanický start;

- návod k obsluze v českém (nebo v anglickém) jazyce v tištěné (nebo elektronické) podobě;

- možnost aktualizace řídícího SW;

- servisní podpora v ČR;

- doprava a zaškolení v českém (nebo v anglickém) jazyce.

Annex No. 1 to the contract: Technical specification of the delivery subject

The subject of the delivery is the equipment: Ultrasonic welding equipment, which allows joining plastic materials using ultrasonic technology. The advantage is lower thermal stress.

It meets the following (minimum) technical parameters and enables the following functions:

- ultrasonic generator frequency 35 kHz;

- power of ultrasonic generator at least 1000 W;

- welding pressure in the range of 30 - 600 N;

- stroke 100 mm;

- touch color display at least 14 ″;

- servo pneumatic cylinder drive (pneumatic cylinder drive with the possibility of setting the start and end position of the cylinder) ;

- welding modes - the possibility of controlling the welding profile according to Energy, Power, Time, Absolute Distance, Depth or a combination of individual modes during one welding profile;

- the possibility of setting the profile with at least three different pressures during the welding process and at least one pressure before and after the welding;

- path measuring system;

- graphical display of the welding profile and the result of the welding process on the color display;

- possibility to save set welding profiles (minimum 200 profiles) ;

- possibility to record the course of the welding process (minimum 200 profiles) ;

- possibility to compare welding processes (minimum 100 processes) ;

- monitoring the quality of the welding process with the possibility of graphical analysis (using CPK metrics and control diagrams);

- Czech communication interface;

- two-hand mechanical start;

- operating instructions in the Czech (or English) language in printed (or electronic) form;

- the possibility of updating the control SW;

- service support in the Czech Republic;

- transport and training in the Czech (or English) language.