High-temperature substrate holder for thin-film deposition

- UHV construction for sputter down deposition;

- DN250ISO-CF stainless-steel mounting flange;

- Ferro-fluidic main rotary feedthrough;

- 4" or 10 cm substrate carrier;

- Substrate carrier heating system (up to 850 °C) compatible with O2 containing working atmosphere (maximum O2 partial pressure of 1 Pa);

- Continuous motorized substrate carrier rotation system (at least 40 rpm maximum speed);

- RF/DC biasing of the substrate carrier while rotating / heating / depositing;

- Adjustable target-to-carrier distance (at least 100 mm distance range) without breaking the vacuum;

- Customized substrate carrier-to-mounting flange distance (will be specified later based on the substrate holder design);

- Power and control unit for heating;

- Power and control unit for rotation;

- Cables and adapters needed for a proper operation.